Hardware Components and Their Roles

Autonomous Cyber-Physical System — D7065E · Dhivya Priya Komathivanan, Luleå University of Technology

The system is an autonomous cyber-physical loop: a controller reads the physical environment through sensors, an agent decides, and the controller acts on the environment through actuators. The hardware below fills four roles — the controller ("brain"), the sensors ("eyes"), the actuators ("hands"), and the supporting parts that keep the brain alive and wire everything together. Two scenarios are supported: Scenario A (real heating control) and Scenario B (fire detection and evacuation, with the fire event modelled in the BuildSim digital twin).

## 1. Controller

| Component | Role in the system |
| --- | --- |
| Raspberry Pi 5 | The controller and brain. Runs Linux, the edge agent and the control loop; reads the sensors and drives the actuators through its GPIO pins. It is the "Board" in the architecture — the boundary between the physical world and the software. |
| Active cooler | Thermal management. The Pi 5 runs hot under continuous load and will slow itself down (throttle) if it overheats; the cooler carries that heat away so the system runs steadily. Support only — it does not take part in the control logic. |

## 2. Sensors — the "eyes"

| Component | Role in the system |
| --- | --- |
| DS18B20 temperature sensor | The primary sensor for Scenario A. Measures room temperature and reports it as a digital value the Pi reads directly. This is the reading the agent perceives and the variable it controls (holding a comfortable band). |
| SEN0570 smoke sensor | The detector for Scenario B. Senses smoke/gas and produces the abnormal signal that triggers the agent’s fire-response playbook. Its output is analog, so it is read through the ADS1115 converter (below). |

## 3. Actuators — the "hands"

| Component | Role in the system |
| --- | --- |
| Grove relay | The actuator for Scenario A. A computer-controlled switch: the Pi sets one GPIO pin on/off to switch the heater, while the relay keeps the Pi electrically isolated from the heater’s higher-power side. This is how the agent physically changes the room temperature. |
| Active buzzer | The alarm actuator for Scenario B. The agent drives it on to sound an audible fire alarm at the bench, giving the twin’s fire response a tangible physical signal on real hardware. |

## 4. Wiring and prototyping

| Component | Role in the system |
| --- | --- |
| Breadboard | The solderless prototyping board. Sensors, actuators and their support parts are pushed into it and joined without soldering; it is the workbench where the circuit is assembled and connected to the Pi. |

## 5. Supporting parts (essential to add)

These do not appear as headline items but are required for the components above to work:

| Component | Role in the system |
| --- | --- |
| 27 W USB-C power supply | Powers the Raspberry Pi 5. The Pi 5 needs its proper supply; an undersized charger causes brownouts and random crashes. |
| microSD card (~32 GB) | The Pi’s storage. It holds the operating system (Raspberry Pi OS) and the project files; the Pi has no built-in storage and boots from this card. |
| Jumper wires (male-female, male-male) | Connect the Pi’s GPIO pins to the breadboard and to each module. Nothing is wired together without them. |
| 4.7 kΩ resistor | Mandatory pull-up resistor on the DS18B20 data line. Without it the temperature sensor cannot communicate reliably. |
| ADS1115 (analog-to-digital converter) | Lets the Pi read the analog smoke sensor. The Pi has no analog input, so the ADS1115 measures the sensor’s voltage and passes a digital value to the Pi over I²C. Without it, the smoke sensor is unreadable. |
| Grove-to-jumper cable | Adapter that connects the Grove relay’s 4-pin connector to the Pi/breadboard. |
| NPN transistor (e.g. 2N2222) + base resistor | Drives the buzzer, which needs more current than a GPIO pin should supply. The Pi switches the transistor, and the transistor passes the full current to the buzzer. |
| Heat source (lamp / heating element) | The load the relay switches in Scenario A. It warms the DS18B20 so that turning the heater on has a real, measurable effect — this is what closes the physical loop. |